SLIMIRON + SIMAC

Design & Architecture Documentation

Version 0.2.2  ·  Hobby Cryptographic Primitive

| Algorithm | Slimiron AEAD (stream cipher + SIMAC MAC) |
| --- | --- |
| Key size | 256-bit |
| Nonce | 96-bit (misuse-resistant via SIV) |
| Tag | 128-bit |
| Security | Birthday bound 2¹²⁸ (256-bit sponge capacity) |
| Rounds | 10 (stream) / 10 (SIMAC) |
| Counter limit | 2³² − 2 blocks ≈ 256 GB per (key, synth_nonce) |
| Status | Hobby project — NOT for production use |

# 1. Overview

Slimiron is a nonce-misuse-resistant Authenticated Encryption with Associated Data (AEAD) scheme built entirely from scratch. It combines two original primitives:

- Slimiron stream cipher — an ARX-based counter-mode stream cipher using the SlimMix permutation.

- SIMAC — a duplex-sponge MAC with a 256-bit capacity region for 128-bit collision resistance.

The design goal is educational: understand every component at the mathematical level, with no external dependencies (not even standard hash functions).

Design Philosophy All initialization constants are derived self-referentially from the Slimiron permutation itself — verified by gen_constants.py using pure Python with no hashlib or external libraries.

# 2. SlimMix — ARX Quarter-Round

SlimMix is the fundamental building block of both the stream cipher and the MAC. It is an Add-Rotate-XOR (ARX) construction operating on four 32-bit words.

## 2.1 Quarter-Round Definition

Given four 32-bit words (a, b, c, d), one SlimMix application performs:

a ← a + b   (mod 2³²)

d ← d ⊕ a

d ← d ⋘ 15

c ← c + d   (mod 2³²)

b ← b ⊕ c

b ← b ⋘ 11

a ← a + b   (mod 2³²)

d ← d ⊕ a

d ← d ⋘ 9

c ← c + d   (mod 2³²)

b ← b ⊕ c

b ← b ⋘ 5

The rotation constants (15, 11, 9, 5) were selected by sweeping 142 candidates and measuring avalanche quality. They satisfy:

- All odd — guarantees invertibility (gcd(rot, 32) = 1 for all odd rotations)

- Non-repeating — no two consecutive rotations are equal

- Full 512-bit avalanche achieved by round 2 (DR2)

## 2.2 Full Permutation

The permutation operates on a 16-word (512-bit) state. Each double-round applies column rounds then diagonal rounds — identical to the ChaCha layout:

for i in 0..(rounds/2):

// Column round

SlimMix(s[ 0], s[ 4], s[ 8], s[12])

SlimMix(s[ 1], s[ 5], s[ 9], s[13])

SlimMix(s[ 2], s[ 6], s[10], s[14])

SlimMix(s[ 3], s[ 7], s[11], s[15])

// Diagonal round

SlimMix(s[ 0], s[ 5], s[10], s[15])

SlimMix(s[ 1], s[ 6], s[11], s[12])

SlimMix(s[ 2], s[ 7], s[ 8], s[13])

SlimMix(s[ 3], s[ 4], s[ 9], s[14])

Default: 10 rounds (5 double-rounds). The column round mixes within columns; the diagonal round mixes across columns, ensuring every output word depends on every input word.

# 3. Slimiron Stream Cipher

## 3.1 State Layout (v0.2.1+)

The 512-bit state is divided into a rate region (words 0–7) and a capacity region (words 8–15):

| Words | Contents | Role |
| --- | --- | --- |
| state[0] | CTR | Counter — incremented each block |
| state[1..3] | N0, N1, N2 | 96-bit nonce (3 × 32-bit LE words) |
| state[4..7] | IV_0..IV_3 | 128-bit initialization vector (derived constant) |
| state[8..15] | K0..K7 | 256-bit key — permanently in capacity, never overwritten |

Security Note Placing the full 256-bit key in state[8..15] (capacity region) means it is NEVER XORed with input/output data and survives all permutation applications intact. Earlier versions incorrectly placed only 128 bits of key, silently discarding K4..K7.

## 3.2 Initialization

state[0]  ← 0                    // counter starts at 0

state[1]  ← LE32(nonce[0..3])

state[2]  ← LE32(nonce[4..7])

state[3]  ← LE32(nonce[8..11])

state[4]  ← IV_0  // slimiron_bootstrap("slimiron-stream-v5")[0]

state[5]  ← IV_1

state[6]  ← IV_2

state[7]  ← IV_3

state[8]  ← LE32(key[0..3])

...

state[15] ← LE32(key[28..31])

## 3.3 Block Generation

Each call to slimiron_block() generates 64 bytes of keystream:

slimiron_block(ctx):

assert ctx.state[0] <= MAX_COUNTER   // 2³² - 2

x ← copy(ctx.state)                 // working copy

x ← permute(x, 10)                  // apply 10-round permutation

for i in 0..16:

x[i] ← x[i] + ctx.state[i]        // add-back (prevents inversion)

stream[i*4..(i+1)*4] ← LE32(x[i])

ctx.state[0]++                       // increment counter

The add-back step (x[i] + state[i]) is critical: it makes inversion of the permutation computationally infeasible even if the output stream is known.

## 3.4 Counter Limit

The counter is capped at SLIMIRON_MAX_COUNTER = 0xFFFFFFFE (2³² − 2). This gives a maximum of (2³² − 1) × 64 bytes ≈ 256 GB per (key, nonce) pair. Exceeding this limit returns -2 and aborts the operation.

# 4. SIMAC — Sponge-Based MAC

## 4.1 Sponge Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| State size | 512 bits (16 × 32-bit words) | Full permutation state |
| Rate (r) | 256 bits (8 words, 32 bytes) | state[0..7] — XORed with input |
| Capacity (c) | 256 bits (8 words, 32 bytes) | state[8..15] — never touched by absorb |
| Birthday bound | 2¹²⁸ | c/2 = 128 bits |
| Rounds | 10 | Per permutation call |

## 4.2 Initialization

SIMAC initialization seeds the capacity region with derived constants XOR'd with the nonce, then immediately permutes the full state to mix key material throughout:

simac_init(ctx, mac_key[32], nonce[12]):

// Rate: load 256-bit mac_key

state[0..7] ← LE32(mac_key[0..31])

// Capacity: derived constants XOR nonce (for per-nonce uniqueness)

state[8]  ← SIMAC_INIT_0 ⊕ LE32(nonce[0..3])

state[9]  ← SIMAC_INIT_1 ⊕ LE32(nonce[4..7])

state[10] ← SIMAC_INIT_2 ⊕ LE32(nonce[8..11])

state[11] ← SIMAC_INIT_3

state[12] ← SIMAC_INIT_4

state[13] ← SIMAC_INIT_5

state[14] ← SIMAC_INIT_6

state[15] ← SIMAC_INIT_7

permute(state, 10)   // mix everything

buffer ← 0[32]

pos ← 0

## 4.3 Absorb

Data is absorbed into the sponge rate region through an internal 32-byte buffer. When the buffer fills, it is compressed into the state:

simac_absorb(ctx, data, len):

// Fill partial buffer first

if ctx.pos > 0:

want ← RATE_BYTES - ctx.pos

if len < want: buffer(ctx.pos..ctx.pos+len) ← data; ctx.pos += len; return

buffer(ctx.pos..) ← data[0..want]

data += want; len -= want

compress(ctx)   // flush buffer into state

// Fast path: full blocks directly (no buffer copy)

while len >= RATE_BYTES:

for i in 0..8: state[i] ⊕= LE32(data[i*4..])

permute(state, 10)

data += 32; len -= 32

// Remaining partial block into buffer

if len > 0: buffer[0..len] ← data; ctx.pos ← len

The fast path bypasses the buffer for full blocks, making bulk absorption efficient. The byte-by-byte and bulk paths produce identical results (verified by test_simac_fast_path).

## 4.4 Padding

Two-marker padding scheme (same as used in Ascon and similar sponge constructions):

simac_pad(ctx):

buffer[pos]          ⊕= 0x01   // start marker

buffer[RATE_BYTES-1] ⊕= 0x80   // end marker

// When pos == RATE_BYTES-1: both bits land on same byte → 0x81

compress(ctx)

This padding is injective: no two distinct (message, length) pairs produce the same padded block, ensuring the MAC is domain-separated from other inputs.

## 4.5 Domain Separation

After each logical field, the caller pads explicitly, then calls simac_domain() to XOR a unique constant into the first capacity word (state[8]) and permute:

simac_domain(ctx, dom):

state[8] ⊕= dom          // only touches capacity — unreachable by absorb path

permute(state, 10)

| Domain | Constant | Label |
| --- | --- | --- |
| SIMAC_DOMAIN_AAD | 0x8439c00f | simac-domain-aad-v5 |
| SIMAC_DOMAIN_CT | 0x35ef9605 | simac-domain-ct-v5 |
| SIMAC_DOMAIN_SIV | 0x493ccf67 | simac-domain-siv-v5 |

Domain Separation Correctness XORing into state[8] (first capacity word) is safe because the absorb path NEVER touches state[8..15]. The domain constant is therefore structurally separate from all absorbed data, preventing any ambiguity between AAD, ciphertext, and SIV inputs.

## 4.6 Finalization

Finalization applies a final padding, XORs a 256-bit mask into the full capacity region, then runs two extra permutations before squeezing the 128-bit tag:

simac_finalize(ctx, tag[16]):

simac_pad(ctx)             // final padding + compress

state[8]  ⊕= SIMAC_FINAL_0

state[9]  ⊕= SIMAC_FINAL_1

...

state[15] ⊕= SIMAC_FINAL_7

permute(state, 10)         // first squeezing permutation

permute(state, 10)         // second squeezing permutation

tag ← LE32(state[0]) || LE32(state[1]) || LE32(state[2]) || LE32(state[3])

The double permutation in finalization provides additional margin: even if one permutation were partially weakened, the second provides a second layer before output is extracted.

# 5. SIV — Synthetic IV (Nonce-Misuse Resistance)

Standard stream ciphers catastrophically fail when a nonce is reused: two ciphertexts encrypted under the same (key, nonce) pair reveal their XOR, leaking plaintext. Slimiron addresses this with a Synthetic IV (SIV) construction.

## 5.1 SIV Derivation

siv_derive(key, nonce, aad, msg) → synth_nonce[12]:

// Derive a dedicated SIV key from the stream cipher

c ← slimiron_init(key, nonce)

siv_key ← slimiron_derive_mac_key(c)    // first 32 bytes of keystream

// Run SIMAC over: original nonce || aad || msg

m ← simac_init(siv_key, nonce)

simac_absorb(m, nonce, 12)              // commit original nonce

simac_absorb(m, aad, aad_len)

simac_absorb_len(m, aad_len)            // length field, big-endian

simac_absorb(m, msg, mlen)

simac_absorb_len(m, mlen)

simac_pad(m)

simac_domain(m, SIMAC_DOMAIN_SIV)

tag ← simac_finalize(m)

synth_nonce ← tag[0..12]               // truncate to 96 bits

## 5.2 SIV Security Properties

| Scenario | Result | Security |
| --- | --- | --- |
| Same key+nonce, same msg+aad | Same synth_nonce | Deterministic (expected) |
| Same key+nonce, different msg | Different synth_nonce | Nonce reuse harmless |
| Different key | Different synth_nonce | Key-binding |
| Tampered synth_nonce | Auth failure (-1) | Integrity protected |

The key insight: the actual encryption uses synth_nonce (derived from the message content) as the nonce, not the user-provided nonce. Even if a user reuses the same nonce for two different messages, the synth_nonces will differ, preventing keystream reuse.

# 6. Full AEAD Construction

## 6.1 Encrypt

slimiron_aead_encrypt(key, nonce, aad, msg) → (synth_nonce, ciphertext, tag):

// Step 1: SIV — derive synthetic nonce from (key, nonce, aad, msg)

synth_nonce ← siv_derive(key, nonce, aad, msg)

// Step 2: Derive 256-bit MAC key from keystream block 0

c ← slimiron_init(key, synth_nonce)

mac_key ← slimiron_derive_mac_key(c)  // keystream bytes 0..31

// counter is now 1

// Step 3: Initialize SIMAC over (mac_key, synth_nonce)

m ← simac_init(mac_key, synth_nonce)

// Step 4: Absorb AAD with domain separation

simac_absorb(m, aad, aad_len)

simac_absorb_len(m, aad_len)      // big-endian 64-bit length

simac_pad(m)

simac_domain(m, SIMAC_DOMAIN_AAD)

// Step 5: Encrypt message + absorb ciphertext into SIMAC

for each 64-byte block i of msg:

keystream_block ← slimiron_block(c)   // counter = i+1

ciphertext[i] ← msg[i] ⊕ keystream_block

simac_absorb(m, ciphertext[i], 64)

// Step 6: Finalize tag

simac_absorb_len(m, mlen)         // ciphertext length, big-endian

simac_pad(m)

simac_domain(m, SIMAC_DOMAIN_CT)

tag ← simac_finalize(m)

// Zeroize all key material

slim_zero(mac_key, 32)

slim_zero(c, sizeof(c))

slim_zero(m, sizeof(m))

## 6.2 Decrypt

slimiron_aead_decrypt(key, synth_nonce, ciphertext, tag, aad) → plaintext | -1:

// Step 1: Derive MAC key (same as encrypt)

c ← slimiron_init(key, synth_nonce)

mac_key ← slimiron_derive_mac_key(c)

// Step 2: Recompute tag over (aad, ciphertext)

m ← simac_init(mac_key, synth_nonce)

simac_absorb(m, aad, aad_len)

simac_absorb_len(m, aad_len)

simac_pad(m)

simac_domain(m, SIMAC_DOMAIN_AAD)

simac_absorb(m, ciphertext, clen)

simac_absorb_len(m, clen)

simac_pad(m)

simac_domain(m, SIMAC_DOMAIN_CT)

calc_tag ← simac_finalize(m)

// Step 3: Verify tag (constant-time)

if crypto_verify_16(calc_tag, tag) != 0:

slim_zero(output, clen)   // always zero output on failure

return -1

// Step 4: Decrypt only after verification

for each 64-byte block i of ciphertext:

keystream_block ← slimiron_block(c)

plaintext[i] ← ciphertext[i] ⊕ keystream_block

Verify-First Decryption Plaintext is NEVER released before the tag is verified. This prevents padding oracle and chosen-ciphertext attacks that exploit partial decryption output.

## 6.3 Wire Format

The on-wire message format is:

| Field | Size | Description |
| --- | --- | --- |
| synth_nonce | 12 bytes | Synthetic nonce (first 12 bytes of SIV tag) |
| ciphertext | mlen bytes | Encrypted message |
| tag | 16 bytes | SIMAC authentication tag |

Total overhead: 28 bytes per message (12 + 16).

# 7. Self-Bootstrapped Constants

All initialization constants are derived from the Slimiron permutation itself — there is no dependency on SHA-256 or any external hash function.

## 7.1 Bootstrap Procedure

slimiron_bootstrap(label: ASCII string) → words[]:

state ← [0] * 16

buffer ← [0] * 32

pos ← 0

// Absorb label bytes into rate region

for byte in label:

buffer[pos] ⊕= byte

pos++

if pos == 32:                    // full block

for i in 0..8: state[i] ⊕= LE32(buffer[i*4..])

state ← permute(state, 10)

buffer ← [0]*32; pos ← 0

// Two-marker padding

buffer[pos]    ⊕= 0x01

buffer[31]     ⊕= 0x80

for i in 0..8: state[i] ⊕= LE32(buffer[i*4..])

state ← permute(state, 10)

return state[0..N]                // squeeze N words

## 7.2 Derived Constants

| Constant | Label | Value (hex) |
| --- | --- | --- |
| SLIMIRON_IV_0 | slimiron-stream-v5 | 0xb9e3ef7f |
| SLIMIRON_IV_1 | slimiron-stream-v5 | 0x7638101d |
| SLIMIRON_IV_2 | slimiron-stream-v5 | 0x53373520 |
| SLIMIRON_IV_3 | slimiron-stream-v5 | 0x654cbc86 |
| SIMAC_INIT_0..7 | simac-init-v5 | 8 words — see header |
| SIMAC_FINAL_0..7 | simac-final-v5 | 8 words — see header |
| SIMAC_DOMAIN_AAD | simac-domain-aad-v5 | 0x8439c00f |
| SIMAC_DOMAIN_CT | simac-domain-ct-v5 | 0x35ef9605 |
| SIMAC_DOMAIN_SIV | simac-domain-siv-v5 | 0x493ccf67 |

The "-v5" suffix in each label distinguishes these from SHA-256-derived constants used in earlier versions. The verifier script gen_constants.py recomputes all constants in pure Python and checks them against the hardcoded values in slimiron.h.

# 8. Security Analysis

## 8.1 Authentication Security

SIMAC's 256-bit capacity gives a birthday-bound collision resistance of 2¹²⁸ MAC computations. An attacker attempting to forge a tag must either:

- Find a collision in the sponge capacity (requires ≈ 2¹²⁸ operations), or

- Invert the SlimMix permutation (computationally infeasible with 10 rounds), or

- Mount a distinguishing attack on the keystream (requires breaking the stream cipher).

## 8.2 Constant-Time Comparison

crypto_verify_16(a[16], b[16]) → 0 | -1:

volatile uint8_t diff ← 0

for i in 0..16: diff |= a[i] ⊕ b[i]

return (diff == 0) ? 0 : -1

The volatile accumulator prevents the compiler from short-circuiting the loop (early exit on first differing byte would leak timing information about the tag position).

## 8.3 Key Material Zeroization

slim_zero(ptr, len):

volatile uint8_t *p ← ptr

while len--: *p++ ← 0

__asm__ volatile("" ::: "memory")   // compiler barrier

The compiler memory barrier prevents Link-Time Optimization (LTO) and Whole-Program Optimization (WPO) from treating the zeroing writes as dead stores. Without this barrier, an optimizing compiler may legally elide the writes since the buffer is never read afterwards.

## 8.4 Limitations

WARNING Slimiron has NOT been cryptanalyzed by independent experts. The SlimMix permutation design is novel and may contain weaknesses not yet discovered. Do not use for any security-sensitive purpose. This is a learning project only.

- No formal security proof

- SlimMix rotation constants selected by empirical avalanche sweep, not differential/linear cryptanalysis

- 10 rounds may be insufficient — ChaCha uses 20 rounds with similar construction

- No side-channel analysis (cache timing, power analysis)

# 9. Performance

## 9.1 Benchmark Results

| Operation | Throughput | Time/call (16 MB) |
| --- | --- | --- |
| Encrypt | 146.51 MB/s | 109.21 ms |
| Decrypt | 252.14 MB/s | 63.46 ms |

Conditions: Intel (scalar unrolled 8×8 XOR path, no AVX2), 16 MB data, 50 iterations, 3 warmup rounds.

Encrypt is slower than decrypt because it requires an extra SIMAC pass over the plaintext for SIV derivation. Decrypt skips this and verifies against the received synth_nonce directly.

## 9.2 Encryption vs Decryption Cost

| Phase | Encrypt | Decrypt |
| --- | --- | --- |
| SIV derivation (SIMAC over plaintext) | Yes (+1 pass) | No |
| MAC key derivation (1 keystream block) | Yes | Yes |
| SIMAC over AAD | Yes | Yes |
| Encrypt/Decrypt blocks (XOR + counter) | Yes | Yes |
| SIMAC over ciphertext | Yes | Yes |
| Tag verification | No | Yes |

## 9.3 XOR Path

Two XOR implementations are compiled conditionally:

- AVX2: Two 256-bit SIMD loads, XOR, stores — 64 bytes in ~2 instructions

- Scalar (fallback): 8× 64-bit integer XOR — unrolled for good pipelining

# 10. Test Suite

| Test | What it verifies |
| --- | --- |
| test_vector | Deterministic output for fixed (key, nonce, msg, aad) |
| test_roundtrip | Encrypt→Decrypt recovers original plaintext |
| test_multiblock | Correct behavior at block boundary sizes (0,1,31,32,33...513) |
| test_avalanche | 1-bit plaintext change flips ≥40 of 128 tag bits |
| test_siv_misuse | Nonce reuse with different msg → different synth_nonce |
| test_forgery | Cipher/tag/synth_nonce tamper → returns -1, output zeroed |
| test_aad | Tampered AAD detected by tag verification |
| test_domain_sep | AAD and ciphertext bytes are not interchangeable in MAC |
| test_wrong_credentials | Wrong key → auth failure; wrong nonce → uses synth_nonce correctly |
| test_empty | Zero-length message and AAD handled correctly |
| test_counter_overflow | slimiron_block() returns -2 when counter > MAX_COUNTER |
| test_slim_zero | slim_zero() fully clears buffer |
| test_verify16 | crypto_verify_16() returns 0/−1 correctly |
| test_simac_fast_path | Bulk absorb == byte-by-byte absorb (identical tags) |
| test_capacity_isolation | Corrupted capacity word → different tag |
| test_xor64 | xor64() matches reference byte-by-byte XOR |
| test_collision | 10,000 random messages produce 0 tag collisions |

# 11. Changelog

| Version | Fix | Type | Description |
| --- | --- | --- | --- |
| v0.2.1 | #1 | PERF | simac_domain() no longer calls simac_pad() internally — saves 2 permutations on both encrypt and decrypt paths |
| v0.2.1 | #2 | SECURITY | slim_zero() compiler barrier prevents LTO/WPO dead-store elimination of key material |
| v0.2.1 | #3 | SECURITY | Full 256-bit key placed in capacity region (state[8..15]); old layout silently discarded K4..K7 — BREAKING |
| v0.2.1 | #4 | COMPAT | simac_absorb_len() now encodes length big-endian (MSB first), matching TLS/HKDF/GCM convention — BREAKING |
| v0.2.2 | #5 | PURITY | All constants derived from Slimiron permutation itself; SHA-256 dependency removed — BREAKING |